/*!

\page create_filling_page Filling Surface with Edges

To generate a \b Filling in the <b>Main Menu</b> select <b>New Entity - > Generation  - > Filling</b>

To create a curving face using several edges you need to define the

following parameters:

\n <b>Edges Compound</b> - the list of edges used for creation of the

surface;

\n \b Minimum and <b>Maximum Degree</b> of equation of the resulting BSpline or Besier curves describing the surface;

\n \b Tolerance for \b 2D and for \b 3D - minimum distance between the

created surface and the reference edge;

\n <b>Number of

Iterations</b> - defines the maximum number of iterations. The iterations are repeated until the required tolerance is reached. So, a greater number of iterations allows producing a better surface.

\n <b>Approximation</b> - if checked, BSpline curves are generated in the process of surface construction (using GeomAPI_PointsToBSplineSurface functionality). By default the surface is created using Besier curves. The usage of <b>Approximation</b> makes the algorithm work slower, but allows building the

surface for rather complex cases.

\n The \b Result of the operation will be a GEOM_Object (face).

\n <b>TUI Command:</b> <em>geompy.MakeFilling(Edges, MinDegree, MaxDegree, Tol2D, Tol3D, NbIter)</em>

\n <b>Arguments:</b> Name + 1 List of edges + 6 Parameters

(Min. degree, Max. degree, Number of iterations, 2D tolerance, 3D

tolerance, Number of iterations, Appro).

\image html filling.png

<b>Example:</b>

\image html filling_compoundsn.png "Initial edges"

\image html fillingsn.png "Resulting surface"

Our <b>TUI Scripts</b> provide you with useful examples of creation of

\ref tui_creation_filling "Complex Geometric Objects".

*/