/*!

\page a1d_meshing_hypo_page 1D Meshing Hypotheses

<br>

<ul>

<li>\ref arithmetic_1d_anchor "Arithmetic 1D"</li>

<li>\ref average_length_anchor "Average Length"</li>

<li>\ref max_length_anchor "Max Size"</li>

<li>\ref deflection_1d_anchor "Deflection 1D"</li>

<li>\ref number_of_segments_anchor "Number of segments"</li>

<li>\ref start_and_end_length_anchor "Start and end length"</li>

<li>\ref automatic_length_anchor "Automatic Length"</li>

<li>\ref fixed_points_1d_anchor "Fixed points 1D"</li>

</ul>

<br>

\anchor arithmetic_1d_anchor

<h2>Arithmetic 1D hypothesis</h2>

<b>Arithmetic 1D</b> hypothesis allows to split edges into segments with a

length that changes in arithmetic progression (Lk = Lk-1 + d)

beginning from a given starting length and up to a given end length.

The direction of the splitting is defined by the orientation of the underlying geometrical edge.

<b>"Reverse Edges"</b> list box allows to specify the edges for which the splitting should be made

in the direction opposing to their orientation. This list box is enabled only if the geometry object

is selected for the meshing. In this case the user can select edges to be reversed either directly

picking them in the 3D viewer or by selecting the edges or groups of edges in the Object browser.

\image html a-arithmetic1d.png

\image html b-ithmetic1d.png "Arithmetic 1D hypothesis - the size of mesh elements gradually increases"

<b>See Also</b> a sample TUI Script of a

\ref tui_1d_arithmetic "Defining Arithmetic 1D hypothesis" operation.

<br>

\anchor deflection_1d_anchor

<h2>Deflection 1D hypothesis</h2>

<b>Deflection 1D</b> hypothesis can be applied for meshing curvilinear edges

composing your geometrical object. It uses only one parameter: the

value of deflection.

\n A geometrical edge is divided into equal segments. The maximum

distance between a point on the edge within a segment and the line

connecting the ends of the segment should not exceed the specified

value of deflection . Then mesh nodes are constructed at end segment

locations and 1D mesh elements are constructed on segments.

\image html a-deflection1d.png

\image html b-flection1d.png "Deflection 1D hypothesis - useful for meshing curvilinear edges"

<b>See Also</b> a sample TUI Script of a

\ref tui_deflection_1d "Defining Deflection 1D hypothesis" operation.

<br>

\anchor average_length_anchor

<h2>Average Length hypothesis</h2>

<b>Average Length</b> hypothesis can be applied for meshing of edges

composing your geometrical object. Definition of this hypothesis

consists of setting the \b length of segments, which will split these

edges, and the \b precision of rounding. The points on the edges

generated by these segments will represent nodes of your mesh.

Later these nodes will be used for meshing of the faces abutting to

these edges.

The \b precision parameter is used to allow rounding a number of

segments, calculated from the edge length and average length of

segment, to the lower integer, if this value outstands from it in

bounds of the precision. Otherwise, the number of segments is rounded

to the higher integer. Use value 0.5 to provide rounding to the

nearest integer, 1.0 for the lower integer, 0.0 for the higher

integer. Default value is 1e-07.

\image html image41.gif

\image html a-averagelength.png

\image html b-erage_length.png "Average length hypothesis - all 1D mesh elements are roughly equal"

<b>See Also</b> a sample TUI Script of a

\ref tui_average_length "Defining Average Length" hypothesis

operation.

<br>\anchor max_length_anchor

<h2>Max Size</h2>

<b>Max Size</b> hypothesis allows splitting geometrical edges into

segments not longer than the given length. Definition of this hypothesis

consists of setting the maximal allowed \b length of segments.

<b>Use preestimated length</b> check box lets you specify \b length

automatically calculated basing on size of your geometrical object,

namely as diagonal of bounding box divided by ten. The divider can be

changed via "Ratio Bounding Box Diagonal / Max Size"

preference parameter.

<b>Use preestimated length</b> check box is enabled only if the

geometrical object has been selected before hypothesis definition.

\image html a-maxsize1d.png

<br>

\anchor number_of_segments_anchor

<h2>Number of segments hypothesis</h2>

<b>Number of segments</b> hypothesis can be applied for meshing of edges

composing your geometrical object. Definition of this hypothesis

consists of setting the number of segments, which will split these

edges. In other words your edges will be split into a definite number

of segments with approximately the same length. The points on the

edges generated by these segments will represent nodes of your

mesh. Later these nodes will be used for meshing of the faces abutting

to these edges.

The direction of the splitting is defined by the orientation of the underlying geometrical edge.

<b>"Reverse Edges"</b> list box allows to specify the edges for which the splitting should be made

in the direction opposing to their orientation. This list box is enabled only if the geometry object

is selected for the meshing. In this case the user can select edges to be reversed either directly

picking them in the 3D viewer or by selecting the edges or groups of edges in the Object browser.

\image html image46.gif

You can set the type of distribution for this hypothesis in the

<b>Hypothesis Construction</b> dialog bog :

\image html a-nbsegments1.png

<br><b>Equidistant Distribution</b> - all segments will have the same

length, you define only the <b>Number of Segments</b>.

<br><b>Scale Distribution</b> - length of segments gradually changes depending on the <b>Scale Factor</b>, which is a ratio of the first segment length to the last segment length.

\image html a-nbsegments2.png

<br><b>Distribution with Table Density</b> - you input a number of

pairs <b>t - F(t)</b>, where \b t ranges from 0 to 1,  and the module computes the

formula, which will rule the change of length of segments and shows

the curve in the plot. You can select the <b>Conversion mode</b> from

\b Exponent and <b>Cut negative</b>.

\image html distributionwithtabledensity.png

<br><b>Distribution with Analytic Density</b> - you input the formula,

which will rule the change of length of segments and the module shows

the curve in the plot.

\image html distributionwithanalyticdensity.png

<b>See Also</b> a sample TUI Script of a

\ref tui_deflection_1d "Defining Number of Segments" hypothesis

operation.

<br>

\anchor start_and_end_length_anchor

<h2>Start and End Length hypothesis</h2>

<b>Start and End Length</b> hypothesis allows to divide a geometrical edge

into segments so that the first and the last segments have a specified

length. The length medium segments changes with automatically chosen

geometric progression. Then mesh nodes are

constructed at segment ends location and 1D mesh elements are

constructed on them.

The direction of the splitting is defined by the orientation of the underlying geometrical edge.

<b>"Reverse Edges"</b> list box allows to specify the edges for which the splitting should be made

in the direction opposing to their orientation. This list box is enabled only if the geometry object

is selected for the meshing. In this case the user can select edges to be reversed either directly

picking them in the 3D viewer or by selecting the edges or groups of edges in the Object browser.

\image html a-startendlength.png

\image html b-art_end_length.png "The lengths of the first and the last segment are strictly defined"

<b>See Also</b> a sample TUI Script of a

\ref tui_start_and_end_length "Defining Start and End Length"

hypothesis operation.

<br>

\anchor automatic_length_anchor

<h2>Automatic Length</h2>

This hypothesis is automatically applied when you select <b>Assign a

set of hypotheses</b> option in Create Mesh menu.

\image html automaticlength.png

The dialog box prompts you to define the quality of the future mesh by

only one parameter, which is \b Fineness, ranging from 0 (coarse mesh,

low number of elements) to 1 (extremely fine mesh, great number of

elements). Compare one and the same object (sphere) meshed with

minimum and maximum value of this parameter.

\image html image147.gif "Example of a very rough mesh. Automatic Length works for 0."

\image html image148.gif "Example of a very fine mesh. Automatic Length works for 1."

<br>

\anchor fixed_points_1d_anchor

<h2>Fixed points 1D hypothesis</h2>

<b>Fixed points 1D</b> hypothesis allows splitting edges through a

set of points parameterized on the edge (from 1 to 0) and a number of segments for each

interval limited by the points.

\image html hypo_fixedpnt_dlg.png

It is possible to check in <b>Same Nb. Segments for all intervals</b>

option and to define one value for all intervals.

The splitting direction is defined by the orientation of the

underlying geometrical edge. <b>"Reverse Edges"</b> list box allows to

specify the edges for which the splitting should be made in the

direction opposite to their orientation. This list box is enabled only

if the geometrical object is selected for meshing. In this case it is

possible to select the edges to be reversed either directly picking them in

the 3D viewer or selecting the edges or groups of edges in the

Object browser.

\image html mesh_fixedpnt.png "Example of a submesh on the edge built using Fixed points 1D hypothesis"

<b>See Also</b> a sample TUI Script of a

\ref tui_fixed_points "Defining Fixed Points" hypothesis operation.

*/