/*!

\page a2d_meshing_hypo_page 2D Meshing Hypotheses

<br>

<ul>

<li>\ref max_element_area_anchor "Max Element Area"</li>

<li>\ref length_from_edges_anchor "Length from Edges"</li>

<li>\ref hypo_quad_params_anchor "Quadrangle parameters"</li>

<li>\ref quadrangle_preference_anchor "Quadrangle Preference"</li>

<li>\ref triangle_preference_anchor "Triangle Preference"</li>

</ul>

<br>

\anchor max_element_area_anchor

<h2>Max Element Area</h2>

<b>Max Element Area</b> hypothesis is applied for meshing of 2D faces

composing your geometrical object. Definition of this hypothesis

consists of setting the <b>maximum area</b> of meshing elements (depending on

the chosen meshing algorithm it can be <b>triangles</b> or <b>quadrangles</b>),

which will compose the mesh of these 2D faces.

\image html a-maxelarea.png

\n

\image html max_el_area.png "In this example, Max. element area is very small compared to the 1D hypothesis"

<b>See Also</b> a sample TUI Script of a

\ref tui_max_element_area "Maximum Element Area" hypothesis

operation.

<br>

\anchor length_from_edges_anchor

<h2>Length from Edges</h2>

<b>Length from edges</b> hypothesis builds 2D mesh segments having a

length calculated as an average edge length for a given wire.

<b>See Also</b> a sample TUI Script of a

\ref tui_length_from_edges "Length from Edges" hypothesis operation.

<br>

\anchor hypo_quad_params_anchor

<h2>Quadrangle parameters</h2>

<b>Quadrangle parameters</b> is a hypothesis for

Quadrangle (Mapping), which allows using this algorithm for meshing of

triangular faces.In this case it is necessary to select the <b>Base vertex</b>

used as a degenerated edge.

\image html hypo_quad_params_1.png "A face built from 3 edges"

\image html hypo_quad_params_res.png "The resulting mesh"

This hypothesis can be also used to mesh a segment of a circular face.

Please, consider that there is a limitation on the selectiion of the degenerated

vertex for the faces built with the angle > 180 degrees (see the picture).

\image html hypo_quad_params_2.png "3/4 of a circular face"

In this case, selection of a wrong vertex for the <b>Quadrangle parameters</b>

hypothesis will generate a wrong mesh. The picture below

shows the good (left) and the bad (right) results of meshing.

\image html hypo_quad_params_res_2.png "The resulting meshes"

<br>

\anchor quadrangle_preference_anchor

<h2>Quadrangle Preference</h2>

This algorithm can be used only together with Quadrangle (Mapping)

algorithm. It allows to build quadrangular meshes even if the number

of nodes at the opposite edges of a meshed face is not equal,

otherwise this mesh will contain some triangular elements.

<br>

This hypothesis has one restriction on its work: the total quantity of

segments on all four sides of the face must be even (divisible by 2).

<br>

\anchor triangle_preference_anchor

<h2>Triangle Preference</h2>

This algorithm can be used only together with Quadrangle (Mapping)

algorithm. It allows to build triangular mesh faces in the refinement

area if the number of nodes at the opposite edges of a meshed face is not equal,

otherwise refinement area will contain some quadrangular elements.

<br>

*/