/*!

\page smeshpy_interface_page Python interface

Python package smesh defines several classes, destined for easy and

clear mesh creation and edition.

Documentation for smesh package is available in two forms:

The <a href="smeshpy_doc/modules.html"> structured

documentation for smesh package</a>, where all methods and

classes are grouped by their functionality, like it is done in the GUI documentation

and the \ref smeshDC "linear documentation for smesh package"

grouped only by classes, declared in the smesh.py file.

The main page of the \ref smeshDC "linear documentation for smesh package"

contains a list of data structures and a list of

functions, provided by the package smesh.py. The first item in

the list of data structures (\ref smeshDC::smeshDC "class smesh")

also represents documentation for the methods of the package smesh.py itself.

The package smesh.py provides an interface to create and handle

meshes. Use it to create an empty mesh or to import it from the data file.

Once a mesh has been created, it is possible to  manage it via its own

methods, described at \ref smeshDC::Mesh "class Mesh" documentation

(it is also accessible by the second item "class Mesh" in the list of data structures).

Class \b Mesh allows assigning algorithms to a mesh.

Please note, that some algorithms, included in the standard SALOME

distribution are always available:

- REGULAR (1D)

- COMPOSITE (1D)

- MEFISTO (2D)

- Quadrangle (2D)

- Hexa(3D)

- etc...

There are also some algorithms, which can be installed optionally,

some of them are based on open-source meshers:

- NETGEN (1D-2D, 2D, 1D-2D-3D, 3D)

... and others are based on commercial meshers:

- GHS3D (3D)

- BLSURF (2D)

To add hypotheses, use the interfaces, provided by the assigned

algorithms.

Below you can see an example of usage of the package smesh for 3d mesh generation.

\anchor example_3d_mesh

<h2>Example of 3d mesh generation with NETGEN:</h2>

\code

from geompy import *

import smesh

###

# Geometry: an assembly of a box, a cylinder and a truncated cone

# meshed with tetrahedral

###

# Define values

name = "ex21_lamp"

cote = 60

section = 20

size = 200

radius_1 = 80

radius_2 = 40

height = 100

# Build a box

box = MakeBox(-cote, -cote, -cote, +cote, +cote, +cote)

# Build a cylinder

pt1 = MakeVertex(0, 0, cote/3)

di1 = MakeVectorDXDYDZ(0, 0, 1)

cyl = MakeCylinder(pt1, di1, section, size)

# Build a truncated cone

pt2 = MakeVertex(0, 0, size)

cone = MakeCone(pt2, di1, radius_1, radius_2, height)

# Fuse

box_cyl = MakeFuse(box, cyl)

piece = MakeFuse(box_cyl, cone)

# Add to the study

addToStudy(piece, name)

# Create a group of faces

group = CreateGroup(piece, ShapeType["FACE"])

group_name = name + "_grp"

addToStudy(group, group_name)

group.SetName(group_name)

# Add faces to the group

faces = SubShapeAllIDs(piece, ShapeType["FACE"])

UnionIDs(group, faces)

###

# Create a mesh

###

# Define a mesh on a geometry

tetra = smesh.Mesh(piece, name)

# Define 1D hypothesis

algo1d = tetra.Segment()

algo1d.LocalLength(10)

# Define 2D hypothesis

algo2d = tetra.Triangle()

algo2d.LengthFromEdges()

# Define 3D hypothesis

algo3d = tetra.Tetrahedron(smesh.NETGEN)

algo3d.MaxElementVolume(100)

# Compute the mesh

tetra.Compute()

# Create a groupe of faces

tetra.Group(group)

\endcode

Examples of Python scripts for all Mesh operations are available by

the following links:

- \subpage tui_creating_meshes_page

- \subpage tui_viewing_meshes_page

- \subpage tui_defining_hypotheses_page

- \subpage tui_quality_controls_page

- \subpage tui_grouping_elements_page

- \subpage tui_modifying_meshes_page

- \subpage tui_transforming_meshes_page

- \subpage tui_notebook_smesh_page

*/